FCC and Gas Plant

The FCC (Fluid Catalytic Cracking) unit is the heart of the modern refinery. This unit is the major producer of gasoline in the refinery. Light olefins and petroleum liquids are also produced by this process, as well as fuel gas and some cycle stocks.

This example is a model of a process for FCC unit, FCC main fractionator and gas plant with absorber oil and heat integration recycle streams. It is intended to:

- Provide an example of how to model the FCC reactor

- Provide an example of how to model the FCC  main fractionator and gas plant columns

- Provide an example of how to model the recycle loops

The model is not intended for equipment design or for specifying other engineering documents without further review by a process engineer with experience of FCC and gas plant process.

The model includes:

- Simple process description.

- Typical process areas: FCC unit, main fractionator, primary absorber, sponge absorber, stripper column, stabilizer column and the main compressor, coolers, separators between these units.

- Key process approach such as how to model the complex main fractionator column.

- Key process control specifications such as FCC calibration parameters, column reflux rate and so on.

With this model, user can vary FCC reactor operating condition and study the impact on petroleum properties of gasoline. User can also manipulate the distillation operating condition to achieve desired amount and quality of gasoline.

With tray sizing utility on distillation column, user can study if downstream separation train is able to handle the changes in FCC feedstock and operating condition.