The refinery-wide model.hsc case illustrates using Petroleum Shift Reactors (short-cut models for refinery reactors) to build a refinery-wide flowsheet. In the basis environment petroleum assays that represent crude using a reasonably granular set of pseudo-components (of the order of 50-70 components) are included.  Two kinds of crudes are considered viz. sweet crudes (Alaska North Slope, Forties Blend and Tiajuana Light) and sour crudes (Arab Light, Arab Heavy and Bachequero). The sweet crudes and sour crudes are processed in separate atmospheric and vacuum distillation units which are modeled using short-cut distillation columns. The main refinery processing units - diesel hydrotreater, kerosene hydrotreater, naphtha hydrotreater, catalytic cracker, hydrocracker, reformer, isomerization and alkylation unit are all built using Petroleum Shift Reactors. These models are based on delta base-shift concept, i.e. each unit is represented by a set of key independent variables (usually feed flowrates and feed qualities) and key dependent variables (usually product flowrates, product qualities, utilities, etc) with their base condition values specified. For conditions different from the base point, the dependent variables are calculated from the independent variables using a set of linear equations whose coefficients are the derivatives of the independents w.r.t. the dependents at the base point. Usually these derivatives are calculated using the rigorous first principles models available in Hysys Petroleum Refining.

This case demonstrates how one could use a refinery-wide model to track materials (e.g. hydrogen), utilities (e.g. steam, fuel, etc), etc. It also shows how one could formulate the overall profit function on a spreadsheet and study refinery performance by manipulating the model degrees of freedom such as FCC conversion and reformer severity.