The Sour Water Strippers

The sour water strippers are widely used in refineries to remove acid gases and ammonia from waste water. The example case file SourWaterStrippers.hsc models a sour water stripping process using two strippers. The first stripper (Hydrogen Sulfide Stripper) is used to remove hydrogen sulfide and carbon dioxide from the waste water. Ammonia is removed in the second stripper (Ammonia Stripper). The stripped water coming out of the bottom of the Ammonia Stripper normally contains less than 50 ppm of ammonia and less than 10 ppm of hydrogen sulfide. The feed flow rate and compositions were specified according to the typical Chevron WWT process conditions [1].

The following reactions are defined in the chemistry:

H2O + CO2 = HCO3- + H3O+

2H2O = OH- + H3O+

H2O + H2S = H3O+ + HS-

HCl + H2O = Cl- + H3O+

H2O + HCO3- = CO3-2 + H3O+

H2O + HS- = H3O+ + S-2

NaOH = Na+ + OH-

H2O + NH3 = OH- + NH4+

[1] Stephen A. Newman, Acid and sour gas treating processes, Gulf Pub. Co., 1985, p734-752